Ghulam Ishaq Khan Institute of Engineering Sciences and Technology (GIKI)

Faculty of Engineering Sciences (FES)

| ## ES 324 | ## Fall 2025 | ## C. E. P. | ## Weight 15 % | ## Due: 16 Dec 2025 |
| --- | --- | --- | --- | --- |

CLO 2: Perform Discrete-Event Simulation and evaluate output data for optimal decision making.

CLO 3: Investigate and analyze real world problems employing computer simulation methods.

CLO 4: Explain the results of an investigation related to a simulation problem/method.

# Objective

The aim of this complex problem is to enable students to understand how to design and simulate a complex real-life problem and to find a solution for improving the system.

# Process

You have samplers that can only sample at a rate of 10 Hz. A signal is received and sampled.  However, the target signal exists at the time given as follows:

- Take the last three digits of your ID. The first digit is the second and the last two digits are the second fractions.

- For example, if the last 3 digits are 102, then the target signal exists at 1.02 seconds.

Clearly, a signal sampled using a 10 Hz sampler cannot detect such a target signal.

- You have to design a signal with an impulse at the time using your ID as given above.

- Next, design a system using the available samplers that can detect any target signal in the hundredth place, not just your target signal. (Hint: You should use delays and an adder to complete the design.)

- Plot the continuous-time signal, the original discrete-time signal (sampled at 10 Hz), and the final discrete-time signal (resampled at 100 Hz).

# Solution Required

Note-1   Make one report containing main body and annexures (any part which is required as annexure, such as the code, should not be part of the main body).

Note-2   This is a team project, with no more than two members per team allowed. Individual projects are also permitted. Each team member will be evaluated individually.

Note-3   The report is to be typo-script and submitted in pdf format. Late submission will not be accepted.

Note-4   The marks from each task will depend on the evaluation conducted at the end of the project. 50% marks will be awarded on successful completion of the tasks. Remaining 50% marks will be awarded based on the individual evaluations.

- Make functional block diagram of the system and explain how your system works.

- Use your ID as an example to solve the problem and show the system works as required.

- Write a Python code to verify your results in part 2. The code should be generic and should work for any target detected within a time range of 10 seconds.

Final submission will be followed by viva. Any student who does not appear for the viva will not be awarded any marks.

TASK 1 (CLO 2) (25 Marks)

Assuming you have no shortage of 10 Hz samplers, as well as other equipment, like delays and adders, design a system that solves the problem. The design should be such that it can be directly simulated as well.

TASK 2 (CLO 3) (25 Marks)

Simulate the solution from Task 1, provide solutions for different scenarios as verification that your solution works.

TASK 3 (CLO 4) (50 Marks)

The project and results are to be submitted in the form of a report as well as a presentation. All group members are to be present during the presentation. Presentation will be followed by individual viva for each group member. At the time of the presentation, the code should be working properly. If the code is not working, results will not be accepted.